Baltic ForBio – A decision support tool for harvesting logging residues

Wenchao Zhou

Department of Forest Economics,

Swedish University of Agricultural Sciences

2021-03-04

Contents

1.	Introduction	2

1.1	Logging residues from final felling	2

1.2	Small trees from pre-commercial thinning	3

1.3	Logging residues from thinning	4

2.	Current version	4

2.1	Logging residues from final felling	5

2.2	Small trees from pre-commercial thinning	8

2.3	Logging residues from thinning	12

3.	FQA	17

4.	References	18

Appendix	18

Appendix 1.1 Calculation of the biomass of logging residues from final felling	18

Appendix 1.2 Estimation of the biomass of small trees from pre-commercial thinning	18

Appendix 1.3 Estimation of the biomass of logging residues from commercial thinning	19

Appendix 1.4 Estimation of average volume of growing trees	19

Appendix 1.5 Calculation of cutting time for small trees in pre-commercial thinning	19

Appendix 1.6 Calculation of forwarding time	19

Appendix 1.7 Calculation of transporting time	21

Appendix 2 Conversion factors	21

Appendix 3 Definitions	21

# Introduction

The Baltic ForBio is a decision support tool for analyzing production costs for harvesting forest residues as energy. Forest residues are in the forms of logging residues from final felling, small trees from pre-commercial thinning, and logging residues from thinning. Logging residues consist of tops and branches of felled trees, usually left in the site after final felling or commercial thinning; small trees are the whole trees removed in pre-commercial thinning. Costs of producing these residues as biomass fuel can vary along supply chain from forest site to end-use plant. For example, costs for utilizing logging residues from final felling can include the cost of forwarding residues to roadside, the cost of chipping the residues at the roadside, and the cost of transporting the chips to a district heating plant. Meanwhile, the cost of forwarding residues can depend on various factors. The time it takes to forward the residues per ha to the roadside depends on the amount of residues per ha, load space of the forwarder used to do the job, transport distance to the roadside, terrain conditions, and so on. For small tree biomass, production costs include the cost of cutting small-sized trees as well. The cutting cost depends on what a cutting method is used. There are two cutting methods in practice: the whole trees method and the delimbed wood method. The time that is consumed to cut small trees depends on the number of trees to cut, average tree size, and so on.

Steps of cost analysis

Generally, the Baltic ForBio estimates forest residues production costs in two steps.

In the first step, the biomass of available residues is estimated for the forest stand that is decided for final felling, pre-commercial thinning, or thinning by using stand-level or individual biomass functions. The user needs providing data describing the stand, e.g., standing volume, share of tree species, etc. In the second step, various costs along residue supply chain are calculated, including costs for harvesting, forwarding, chipping, and transporting. The user needs to provide relevant technical data, e.g., forwarder’s load space, economic data, e.g., forwarder cost per hour, and spatial data, e.g., single-way distance to roadside, transportation distance to a district heating plant. Revenues from producing residues can be calculated, provided that the price for selling the biomass is provided. Finally, the Baltic ForBio can generate a cost analysis report, which can be saved as pdf file or printed.

## Logging residues from final felling

Logging residues include tops and branches of felled trees. Basic biomass supply chains are:

- Logging residues - Forwarding residues to roadside

- Logging residues - Forwarding residues to roadside - Chipping residues at roadside - Transporting chips to plant

- Logging residues - Forwarding residues to roadside – Transporting loose residues to plant

The amount of tops and branches is estimated using Lehonen (2004)’s stand-level biomass functions for pine, spruce and broadleaf, respectively. The functions that provide the biomass of components living branches and stems from stem volume, or standing volume, are used. The biomass of tree tops is assumed to be 5% of the stem biomass. The time consumed by forwarding residues to roadside is estimated using time consumption functions for forwarding logging residues (see Appendix 1.6). The time consumed by transporting chips or loose residues is estimated using time consumption functions for transporting (see Appendix 1.7).

Example 1.1

A four ha spruce stand at 90 years old is located in middle Sweden and planned for final felling. The standing volume is measured to be 375 m3/ha, consisting of pine 0%, spruce 100% and broadleaf 0%. Available logging residues are estimated (by the App) to be 26.4 ton DM/ha or 57.9 m3 /ha solid volume. 75% of the residues are extracted for energy use. Average forwarding distance to roadside is 400 meters. When a forwarder with load space 16 m3 solid is used, the forwarding time consumption is estimated (by the App) to be 3.5 G15-h per ha. Given 950 kr/G15-h forwarder cost, the cost for forwarding is estimated to be 3299 kr per ha, and 13196 kr for the whole stand.

## Small trees from pre-commercial thinning

The entire biomass of small trees removed is recovered as energy. Basic biomass supply chains are:

- Cutting small trees - Forwarding to roadside

- Cutting small trees - Forwarding to roadside - Chipping at roadside - Transporting chips to plant

- Cutting small tress - Forwarding to roadside - Transporting loose residues to plant

The biomass of small trees is estimated by using individual tree biomass functions of Marklund (1988). The biomass functions provide the biomass of components living branches, dead branches and stems, for pine, spruce, and broadleaf, respectively. The biomass is a function of the tree’s diameter at breast height. Production costs can include the cost of cutting trees, the cost of forwarding the removed trees to roadside, or chipping and transporting costs. Either standard machine or small machine can be used to perform the cutting operation. When small harvester is selected, the user needs to provide the harvester’s productivity. When standard harvester is used, there are two cutting methods to choose: the whole trees method and the delimbed wood method. The time consumption of cutting small trees is estimated using time consumption function (Appendix 1.5); the time consumption of forwarding the biomass to roadside is estimated using time consumption function (Appendix 1.6); the time consumption of transporting the biomass to plant is estimated using time consumption function (Appendix 1.7).

Example 1.2

There is a two ha of forest stand at 25 years old within which a pre-commercial thinning is decided to carry out. Small trees to remove are recovered as bioenergy. The stand has 5000 trees per ha, consisting of spruce 40% and 60% broadleaf trees. The mean diameter (measured in quadratic mean diameter) of the stand is 6.8 cm. The manager plans to cut 35% of the stems, resulting in 3250 tree per ha left in the stand. It is required that the mean diameter of the remaining stand is 6.8 cm, and that the stand after thinning consists of 40% spruce and 60% broadleaf trees. It can be derived that 1750 stems per ha are cut, with a mean diameter of 6.8 cm. The biomass of the small trees removed is estimated (by the App) to be 16.1 ton DM/ha, or 35.3 m3 /ha solid volume. Thinning is carried out using standard machine, together with the whole trees cutting method. Accordingly, the harvesting time is estimated (by the App) to be 21.3 G15-h per ha. Provided that the harvester cost is 1050 kr/G15-h, the harvesting cost is estimated to be 22319 kr per ha and 44638 kr for the whole stand. The average distance to the roadside is 400 meters, and the forwarding time is estimated (by the App) to be 4.0 G15-h per ha. Given a forwarder cost of 800 kr/G15-h, the forwarding cost is estimated to be 3239 kr per ha, and 6478 kr for the whole stand.

## Logging residues from thinning

Logging residues from commercial thinning include tops and branches of harvested trees, which usually are left on the site. Basic biomass supply chains are:

- Logging residues - Forwarding residues to roadside

- Logging residues - Forwarding residues to roadside - Chipping at roadside - Transporting chips to plant

- Logging residues - Forwarding residues to roadside - Transporting loose residues to plant

Different to biomass estimation of logging residues from final felling, the biomass of residues from thinning is estimated using the individual tree biomass functions of Marklund (1988). Using diameter at breast height of a tree as independent variable, the biomass functions of Marklund (1988) provide the biomass components branches and stems. In calculating the total amount of logging residues (tops and branches), the biomass of top part is assumed to be 5% of the tree’s stem biomass. The time consumption of forwarding biomass to roadside is estimated using time consumption function (Appendix 1.6); the time consumption of transporting biomass to plant is estimated using time consumption function (Appendix 1.7).

Example 1.3

There is a two ha forest stand at 35 years old in Southern Sweden. The stand has 2000 trees per ha, with pine 0%, spruce 80% and broadleaf 20%. The mean diameter (measured in quadratic mean diameter) of the stand is 12.2 cm. A thinning operation is decided to carry out for the stand, and the logging residues (tops and braches) will be extracted for energy use. Thinning intensity is 35%. After thinning, 1300 trees per ha are left, with spruce 80% and 20% broadleaf. The mean diameter (measured in quadratic mean diameter) of the stand after thinning is 13.2 cm. It can be derived that the number of stems to remove is 700 stem/ha. The biomass of available logging residues is estimated (by the App) to be 4.9 ton DM/ha, or 10.7 m3 /ha solid volume. Residues are forwarded to the roadside, with average distance 400 meters. The forwarding time is estimated (by the APP) to be 1.6 G15-h per ha. Given a forwarder cost of 800 kr/G15-h per ha, the forwarding cost is estimated (by the App) to be 1286 kr per ha, or 2572 kr for the entire stand.

# Current version

System requirements

The Baltic ForBio app requires Microsoft windows 10 and .Net framework 4.6 or higher. The app installer can check to see if the framework exists, and if not, it can start to download the framework from internet and install it.

The Baltic ForBio starts with a welcome page (Figure 1), allowing the user to choose a country.  Press button Start, the main page shows up. Note that if the user does not make a country selection, the country saved in the previous run is chosen automatically.

Figure 1

In the main page, the user is waited for selecting which type of biomass (forest residues) for cost analysis. There are three options: 1) Logging residues from final felling, 2) Small trees from pre-commercial thinning, and 3) Logging residues from thinning.

## Logging residues from final felling

To perform cost analysis for harvesting logging residues generated at final felling, select Logging residues from final felling in box Biomass Type in the top of the page (Figure 2). Then, in Tab page Stand and Biomass, provide stand-level data describing the stand. This includes the following information. Property name - name for the stand (optional); Area - size of the stand to harvest; Age - the age of the stand for final felling; Standing volume - the growing stock measured in cubic meter per ha standing volume; Species share - percent volume distribution by tree species. While selecting or typing, the App estimates the amount of biomass residues (tree tops and branches) generated at final felling; the estimated biomass is provided in the table on the right-hand side of the page, where the biomass is expressed in three different units. In box Extraction Rate the user needs to specify what percent of the residues is going to extract for energy use (note: Swedish forestry agency recommends 20% of available logging residuals shall be left at the site for biodiversity protection and nutrition purpose). Once all the required information is filled, the user can either select the tab Production Method and Costs or press the button Next (at the bottom of page), switch to Tab page Production Method and Costs.

In page Production Method and Costs (Figure 3), the user needs to provide technical, spatial and economic data necessary for calculating various costs of harvesting logging residues.

Figure 2

In box Forwarding the user can input information regarding forwarding the logging residues to the roadside. Load volume is the capacity size of the forwarder, Forwarding distance is the average distance in meters from the center of the logging site to the roadside; Forwarder cost is the per hour cost for forwarding. Forwarding time (in gross effective hour, G15-h, note: G0-h/G15-h = 0.92, ref: Olovsson, 2014) is the time consumed by forwarding activity and is calculated by the App.

In box Transportation Options, choose where biomass residues are paid and transportation method. Roadside, the residues are paid at the roadside without further transportation. This option accounts for only forwarding cost; Plant with chips, the residues are chipped at the roadside, then chips are further transported by chip truck to a plant, at which the biomass is paid. This option accounts for forwarding cost, chipping cost and transportation cost. Additionally, data on Distance to plant gate, Transporter cost, and Chipping cost are required to specify; Plant with loose residues, the biomass is further transported in the form of loose residues to a plant at which the biomass is paid. This option accounts for forwarding cost and transportation cost. Additional data on Distance to plant gate and transporter cost are required in order to calculate the transporting cost. Transporting time (in hours) consumed by transporting the residues from the roadside to the plant is calculated by the App.

Figure 3

In box Biomass Price, the user can give the price for the biomass corresponding to the chosen transport method in order to obtain the Revenue from selling the biomass. When the price is not known, leave it be zero, then revenue will not be calculated.

In box Costs and Revenue is a summary provided. Costs may include harvesting cost (which is zero, when Logging residues from final felling is chosen), forwarding cost, chipping cost, and transporting cost. The sum of these costs is given in row subtotal. The net revenue is revenue minus total cost. In the table the first column shows values in per ha, while the second column shows the values for the whole stand.

Press button Generate Report, a report page will be generated and Tab page Report is open. The report provides three tables (Figure 4). The first table summarizes the revenue and costs; the second presents the stand data; the third presents the technical and economic data. The user can save the report in a pdf file, or print it to paper.

Figure 4

## Small trees from pre-commercial thinning

All the biomass of small trees from pre-commerical thinning is recovered for bioenergy use. The production costs can include cutting the small trees, forwarding the removed trees to the roadside, and/or chipping and transporting the biomass further to a plant. Harvest machine includes standard machine or small machine. When standard machine is used, alternative cutting methods include whole trees method and delimbed wood method.

To perform cost analysis, select Small trees from pre-commercial thinning in box Biomass Type in the top of the page (Figure 5). In Tab page Stand and Biomass, provide stand-level data describing the stand to carry out pre-commercial thinning. This includes the following information. Property name - name for the stand (optional); Area - size of the stand; Age - the age of the stand. Data decribing the stand before and after thinning is required. In box Stems, Thinning intensity - the ratio of the removed stem number to the stem number before thinning; Stem per ha - the total stem number of the stand per ha before thinning in column Before, and the stem numbe after thnning in column After. Species share - the stem share of each tree species in the total stem number before and after thinning. Mean diameter is the diamter of the stand before and after thinning, where qudratic mean diameter (which is correspoing to the diameter of a tree with mean basal area) should be used.

The biomass of the removed small tress is shown in the table on the right-hand side, where the biomass is expressed in three different units. It is assumed that all the biomass is recovered for bioenergy use.

Once all the required information is filled, the user can either select the tab Production Method and Costs or press the button Next (at the bottom of page), switch to page Production Method and Costs.

In page Production Method and Costs (Figure 6), in box Harvesting, the user needs to choose which type of harvest machine from standard machine or small machine. If small machne is selected, the user is asked to input the productivity of the machine, i.e., the solid volume harvested per working hour by the machine. If standard machine is chosen, additional information is required. Which type of Cutting Method is used either the whole trees method or the delimbed wood method. Harvester cost is per hour cost and needs input by the user. Harvesting time is the time consumption of cutting the small trees and is calculated by the App.

In box Forwarding the user can input information regarding forwarding the harvested small trees to the roadside. Load volume is the capacity size of the forwarder, Forwarding distance is the average distance in meter from the center of the logging site to the roadside; Forwarder cost is the per hour cost for forwarding. Forwarding time is the time consumed by forwarding activity and is calculated by the app.

In box Transportation Options, choose where the biomass residues are paid and transportation method. Roadside, the residues are paid at the roadside without further transportation. This option accounts for only forwarding cost; Plant with chips, the residues are chipped at the roadside, then chips are further transported by chip truck to a plant, at which the biomass is paid. This option accounts for forwarding cost, chipping cost and transportation cost. Additional data on Distance to plant gate, Transporter cost, and Chipping cost are required to specify; Plant with loose residues, the biomass is further transported in the form of loose residues to a plant and is paid at the plant gate. This option accounts for forwarding cost and transportation cost. Data on Distance to plant gate and transporter cost are required in order to calculate the transporting cost. Transporting time (in hours) consumed by transporting the residues from the roadside to the plant gate is calculated by the App.

In box Biomass Price, the user can give the price for the biomass corresponding to the chosen transport method in order to obtain the Revenue from selling the biomass. When the price is not known, leave it be zero, then revenue will not be calculated.

In box Costs and Revenue is a summary provided. The costs may include harvesting cost, forwarding cost, chipping cost and transporting cost. The sum of these costs is given in row subtotal. The net revenue is revenue minus the total cost. In the table the first column shows the values in per ha, while the second column shows the values for the whole stand.

Press button Generate Report, a report page will be generated and the tab page Report is open. The report provides three tables (Figure 7). The first table summarizes the revenue and costs; the second presents the stand data; the third presents the technical and economic data. The user can save the report in a pdf file, or print it to paper.

Figure 5

Figure 6

Figure 7

## Logging residues from thinning

Residues from commercial thinning is the biomass that consists of tree tops and branches from the removed trees, usually left in the site after thinning. Costs of extracting such residues can include the cost of forwarding the residues to the roadside, and/or chiping the residues at the roadside, and transporting the biomass further to a plant. Data decribing the stand before and after thinning action is required.

To perform cost analysis, select Logging residues from thinning in box Biomass Type in the top of the page (Figure 8). In Tab page Stand and Biomass, provide stand-level data describing the stand for thinning. This includes the following information. Property name - name for the stand (optional); Area - size of the stand; Age - the age of the stand. Data decribing the stand before and after thinning is required. In box Stems, Thinning intensity - the ratio of the removed stem number to the stem number before thinning; Stem per ha - the total stem number of the stand per ha before thinning in column Before, and the stem numbe after thnning in column After. Species share - the stem share of each tree species in the total stem number before and after thinning. Mean diameter is the diamter of the stand before and after thinning, where qudratic mean diameter (which is correspoing to the diameter of a tree with mean basal area) should be used.

The biomass of residues is provided in box Logging Residues. It is assumed all the biomass is extracted for bioenergy use.

Once all the required information is filled, the user can either select the tab Production Method and Costs or press the button Next (at the bottom of page), switch to page Production Method and Costs.

In page Production Method and Costs (Figure 9), in box Forwarding the user can input information regarding forwarding the biomass residues to the roadside. Load volume is the capacity size of the forwarder, Forwarding distance is the average distance in meter from the center of the logging site to the roadside; Forwarder cost is the per hour cost for forwarding. Forwarding time is the time consumed by forwarding activity and is calculated by the App.

In box Transportation Options, choose where the biomass residues are paid and transportation method. Roadside, the residues are paid at the roadside without further transportation. This option accounts for only forwarding cost; Plant with chips, the residues are chipped at the roadside, then chips are further transported by chip truck to a plant, at which the biomass is paid. This option accounts for forwarding cost, chipping cost and transportation cost. Data on Distance to plant gate, Transporter cost, and Chipping cost are required to specify; Plant with loose residues, the residues are further transported in the form of loose residues to a plant and is paid at the plant gate. This option accounts for forwarding cost and transportation cost. Data on Distance to plant gate and transporter cost are required in order to calculate the transporting cost. Transporting time is the hours consumed by transporting the residues from the roadside to the plant and is calculated by the App.

In box Biomass Price, the user can give the price for the biomass corresponding to the chosen transportation method in order to obtain the Revenue from selling the biomass. When the price is unknown, leave it be zero, and revenue will not be calculated.

In box Costs and Revenue is a summary provided. The costs may include forwarding cost, chipping cost and transporting cost. Row subtotal shows the sum of these costs. The net revenue is revenue minus total cost. In the table the first column shows the values in per ha, while the second column shows the values for the whole stand.

Press the button Generate Report, a report page is generated and the tab Report is open. The report provides three tables (Figure 10). Table 1 is the revenue and cost table, table 2 presents the stand data, table 3 presents the technical and economic data. The user can save the report in a pdf file, or print it to paper.

Figure 8

Figure 9

Figure 10

# FQA

  - How can I start a cost analysis for harvesting forest residues using default values set by the Baltic ForBio?

Choose File > New. Then, all are set to default values.

  - How can I save my current work to a file?

To save your current data, choose File > Save. By default, the property name, if you provided, is chosen as a file name. Anyhow, you can always type a name.

  - How can I use my saved data?

To start an anlasis with your data previously saved, choose File > Open. Then find the file you saved before.

  - What happens if I quit the App by choosing File > Exit?

If you close the App by clicking Exit, the App will automatically save your current work. The advantage is that the App can recover your work, when you run it next time without choosing a different country.

  - What happens if I skip selecting a country at the welcome page?

In this case, the App will recover the work when you quit the App by clicking Exit previously.

  - What happens if I quit the App by choosing File > Exit and Clear?

In this case, you close the App without saving your current work. Moreover, the App will clean the work and set all to default values.

  - Is it possible to disable the welcome page?

Yes. At the bottom-right corner of the main window, uncheck the splash box, the welcome page will not show up next time when you run the App.

  - How can I enable the welcome page?

At the bottom-right corner of main window, check the splash box, then the welcome page will show up next time when you run the App.

  - Can I update the default values in the tool?

Yes.

Choose Help > Configuration File Manager > Select a Country

Click button Download to download the Configuration File for the country, and save the file.

Edit the values in the download xml file using Notepad or WordPad (Note: you can change only values. Any others like unit are not allowed to change). Save the file.

Click button Upload to update the configuration file.

# References

Brandel, G., 1990. Volymfunktioner för enskilda träd. Tall, gran och björk. SLU. Inst. f. skogsproduktion. Rapport Nr 26. Garpenberg.

Lehtonen, A., R. Mäkipää, J. Heikkinen, R. Sievänen and J. Liski, 2004. Biomass expansion factors (BEFs) for Scots pine, Norway spruce and birch according to stand age for boreal forests. Forest Ecology and Management 188(1): 211-224.

Marklund, L.G., 1988. Biomassafunktioner för tall, gran och björk i Sverige. Sveriges Lantbruksuniversitet, Rapporter-Skog 45, 1–73.

Olovsson, Jonas, 2014. Effekten av aggregattyp och skördad medelstamvolym på skördarens produktivitet och ekonomi i slutavverkning. Second cycle, A2E. Department of Forest Biomaterials and Technology, SLU, Sweden.

Ram P. Sharma and Johannes Breidenbach, 2015. Modeling height-diameter relationships for Norway spruce, Scots pine, and downy birch using Norwegian national forest inventory data, Forest Science and Technology, 11(1): 44-53.

# Appendix

## Appendix 1.1 Calculation of the biomass of logging residues from final felling

Logging residues from final felling consist of branches and tops, needles and stumps. The amount is estimated using the stand-level biomass function for tree components of Lehtonen etc al. (2004). In calculation, only the biomass of tops and branches is accounted for. The biomass of branches and stems is estimated based on standing volume and using function (3) of Lehtonen etc al. (2004), where components’ biomass is a function of stem volume. The tops biomass is assumed to be 5% of the stem biomass.

## Appendix 1.2 Estimation of the biomass of small trees from pre-commercial thinning

To calculate the biomass of removed small trees, individual tree biomass functions of Marklund (1988) are used. The tree biomass consists of branches, tops and stems. The biomass of branches and stems is estimated from tree’s diameter, separately. The amount of tree’s top biomass is assumed to be 5% of its stem biomass. The mean diameter of removed trees is calculated by using the following formula:

dh=db2-(1-θ)da2θ

Where db and da are quadratic mean diameter (i.e., dg, basal area mean diameter) of the stand before and after thinning, respectively. θ is thinning intensity, the ratio of the stem number of removed trees to the stem number before thinning. For pine trees, Marklund (1988)’s functions T13, T17, T21 and T1 are used for living branches including needles, needles, dead braches, and stem, respectively; For spruces trees, Marklund (1988)’s functions G11, G15, G19 and G1 are used for living branches including needles, needles, dead branches,  and stem, respectively; for broadleaf trees, Marklund (1988)’s functions B11, B15 and B1 are used for living branches, dead branches, and stem, respectively.

## Appendix 1.3 Estimation of the biomass of logging residues from commercial thinning

The biomass of logging residues from thinning is estimated by using the same function of Marklund (1988) as for small trees from pre-commercial thinning.

## Appendix 1.4 Estimation of average volume of growing trees

In calculating time consumption of cutting small trees, the single tree volume functions of Brandel (1990) are used to calculate the average volume of removed trees. We use functions (nr: 100-01) with two independent variables diameter and height. The height of trees is estimated by using height-diameter relationship function of Sharma and Breidenbach (2015), where OLS estimation of equation 4 is used.

## Appendix 1.5 Calculation of cutting time for small trees in pre-commercial thinning

Function for harvesting small trees

T = K * 1.15 * (7930 / UT + 220 * V + 45),

where   T = time consumption, G15 - cmin/tree

UT = number of removed stems per ha

V = average stem volume

K = 0.90 for whole trees method, 0.75 for delimbed wood

Productivity in G15-h/m3f

P = 1 / (6000 / T * V * (1 + 0.01 * P1)),

where  P1 = -6.7 + 0.025 * UT,	when the whole trees cutting method is used,

P1 = 0, when the delimed wood method is used.

## Appendix 1.6 Calculation of forwarding time

Time consumption of forwarding is divided into two parts: terminal time and driving time.

Forwarding logging residues from final felling

Function for terminal time:

T = (65 + 3.79 * UT) * K1 / UT,

where  	T = time consumption, G15-min/m3f,

UT = Volume of logging residues, m3f/ha,

K1 = 0.75

Function for driving time

T1 = (2 * A) / (H * L)

where   T1 = time consumption, G15-min/m3f,

A = single way distance to roadside, m,

H = driving speed, in m/G15-min. We use H = 60 m/G15-min in the calculation.

L = load size, m3f

Productivity in G15-h/m3f

p = (T + T1) / 60

Forwarding small trees from pre-commercial thinning

Function for terminal time

T1 = 1.1 * (53 + 2.46 * UT)  	when the whole trees method is used for cutting

T1 = 1.1 * (53 + 2.84 * UT)   	when the delimbed wood method is used for cutting

UT = 0.006 + 1.3 * V

where 	T1 = G15 - min/ha

V = Volume of small trees, m3fub,

UT = Volume of small trees, m3fbio per ha

Function for driving time

T = (2 * A) / (50 * LK);

where   T = G15-min/m3fbio,

LK = load size, equal to 6.5 with whole trees, while 9.5 with delimbed wood,

A = single way forwarding distance, m

Productivity in G15-h/m3fbiomass

P = (T1 / UT + T) / 60;

Forwarding residues from commercial thinning

Function for terminal time

T1 = 1.1 * (53 + 2.84 * UT),

UT = 0.006 + 1.3 * V,

where  T1 = G15 - min/ha,

UT = volume of residues in m3fbio per ha,

V = volume of residues, m3fub

Function for driving time

T = (2 * A) / (50 * LK)

Where	T = G15-min/m3fbio

A = single way forwarding distance, m,

LK = Load size, m3f. We use LK = 9.5 m3f in the calculation.

Productivity in G15-h/m3fbiomass

P = (T1/UT + T) / 60

## Appendix 1.7 Calculation of transporting time

The time spending on transporting chipped residues or loose residues is estimated using the following formula:

2*DS*UT/L

Where D is distance in km from roadside to plant, S is speed of transport vehicle in km/hour, UT is amount of residues in m3 solid volume, and L is load space of the vehicle in m3 solid volume.

# Appendix 2 Conversion factors

|  | ton DM | MWH | m3 loose | m3 solid |
| --- | --- | --- | --- | --- |
| Logging residues | 1 | 4.812 | 15.664 | 2.193 |
| Whole trees | 1 | 4.771 | 10.823 | 2.381 |

Source: https://www.skogskunskap.se/rakna-med-verktyg/skogsbransle/wecalc---energisnurran/

# Appendix 3 Definitions

List of term definitions

| G0-h | Hour of productive machine time without breaking. G0-h/G15-h = 0.92 |
| --- | --- |
| G15-h | Hour of productive machine time with breaking time less than 15 minutes. |
| Logging residues | Tops and branches of felled trees in final felling or commercial thinning, usually left in the forest site. |
| Mean diameter | The mean diameter in the term of quadratic mean diameter of the trees in a forest stand, which corresponds to the diameter of a tree with average basal area. |
| Small trees | Small-sized trees removed in pre-commercial thinning. |
| Thinning intensity | The ratio of removed stem number to the stem number before thinning. |